# SIMATS ENGINEERING

## CO3- AT2 - Design Task - Medium

### COURSE NAME: Advance Wireless Communication Systems / 5G / 6G Communication Systems

COURSE CODE: ECA56

### COURSE OUTCOME COVERED

CO: 3 - Design spectrum access strategies, waveform generation methods, and multiple access techniques for efficient wireless transmission systems. (BL6)

Assessment Tool Number	: CO3_AT2 Assessment Tool Weightage	: 35% Assessment Tool	: Design Task

Duration	: 180 Minutes

Marks	: 100 Marks

Type of Assessment Conduction : At Home Evaluation Type	: GitHub

Result Generation	: Students design and justify appropriate wireless

communication solutions by applying engineering principles to meet specified requirements.

Topic Covered	:

Spectrum management, licensed/unlicensed spectrum, dynamic spectrum access, cognitive radio, OFDMA, resource block allocation, GFDMA, NOMA, power- domain multiplexing, SIC, and IoT applications. Advanced waveforms and wireless technologies including GFDM, FBMC, UFMC, mmWave propagation, path loss and penetration, millimeter-wave antenna systems, MIMO, and Massive MIMO fundamentals.

Short description about assessment: Students design a system / model based on given requirements to assess creativity and application skills. This question paper consists of a total of 40 questions, out of which each student is required to answer any 2 questions. Each question carries 50 marks, and the paper carries a total of 100 marks

QUESTIONS: 2 (2 X 50 = 100)	Total Marks: 100	Duration: 3

Code of Conduct:

I 	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

### RUBRICS FOR CALCULATION:

| Design Task |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightag e | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Design | 20% | Demonstrates deep understanding of design objectives, constraints, and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Lacks understandin g of the design context |
| Evaluation of Design | 25% | Provides in-depth critique identifying strengths, weaknesses, and design gaps | Identifies key issues with reasonable analysis | Limited critique; mostly descriptive feedback | No meaningful critique provided |
| Justification | 20% | Supports critique with strong reasoning, evidence, and relevant principles | Provides justification with some supporting evidence | Weak or unclear justification | No justification for feedback |
| Suggestions for Improvement | 20% | Offers innovative, practical, and well-justified improvement suggestions | Provides useful suggestions with minor gaps | Suggestions are limited or partially relevant | No constructive suggestions provided |
| Communicatio n | 15% | Communicates feedback clearly, professionally, and engages actively in discussion | Communicates well with minor issues | Communication lacks clarity or engagement | Poor communicatio n and minimal participation |

- System Architecture for Multi-access Edge Computing (MEC) to Reduce Latency in Smart Traffic Applications

Objective

To design a Multi-access Edge Computing (MEC) architecture that processes traffic data close to vehicles and roadside devices, thereby reducing communication latency and enabling real-time traffic management.

Working

Data Collection:

Cameras, traffic sensors, GPS devices and connected vehicles collect traffic information such as vehicle speed, density and road conditions.

5G/V2X Communication:

The collected data is transmitted through 5G or V2X communication to the nearest MEC server.

Edge Processing:

Instead of sending all data to a distant cloud, the MEC server processes the information locally.

Real-Time AI Analysis:

AI/ML algorithms at the edge detect traffic congestion, accidents and abnormal traffic conditions.

Traffic Decision:

The MEC server can determine whether traffic signals should be changed, alternative routes suggested, or emergency vehicles given priority.

Fast Response:

Decisions are sent immediately to traffic controllers, RSUs and vehicles.

Cloud Coordination:

Important or historical data is forwarded to the cloud for long-term storage, large-scale analytics and AI model training.

Advantages

Low communication latency

Real-time accident detection

Faster traffic signal control

Reduced cloud bandwidth consumption

Improved road safety

Better congestion management

Supports autonomous and connected vehicles

Scalable for smart-city applications

Conclusion

The proposed MEC architecture places computation near the traffic sources. 5G/V2X + MEC + IoT + AI enables rapid processing and decision-making, making it suitable for intelligent traffic signals, accident detection and connected-vehicle applications.

- Cloud-Edge Communication Model for Wireless Healthcare Monitoring

Objective

To develop a cloud-edge communication model that enables continuous monitoring of patients using wireless sensors while providing fast local responses and cloud-based medical analysis.

Working

Wireless Data Collection:

Wearable sensors continuously measure parameters such as heart rate, SpO₂, body temperature and blood pressure.

Patient Gateway:

Sensor data is transferred using Bluetooth, Wi-Fi or another wireless connection to a smartphone or IoT gateway.

Edge Processing:

The nearby edge server performs immediate processing of the patient's data.

Abnormality Detection:

Edge AI analyzes the data and detects potentially abnormal conditions.

Emergency Response:

If a critical condition is detected, an alert can be sent immediately to medical staff or caregivers without waiting for cloud processing.

Cloud Communication:

Patient data and selected results are transmitted to the cloud through the Internet/5G.

Cloud Analytics:

The cloud stores historical patient information and performs large-scale analytics and AI/ML model training.

Doctor/Healthcare Access:

Authorized healthcare professionals can access patient information through a secure web or mobile application.

Cloud vs Edge Processing

Parameter

Edge

Cloud

Processing location

Near patient

Remote data center

Response time

Very low

Higher

Real-time alerts

Excellent

Depends on network

Long-term storage

Limited

High

Large-scale analytics

Limited

Excellent

Bandwidth requirement

Lower

Higher

Advantages

Real-time patient monitoring

Faster emergency alerts

Reduced network latency

Reduced cloud bandwidth

Continuous remote monitoring

Historical data storage in cloud

AI-based abnormality detection

Better scalability for multiple patients

Security Requirements

Because healthcare data is sensitive, the system should use:

Authentication

Encryption during communication

Secure patient identification

Access control

Secure cloud storage

Privacy-preserving data processing

Conclusion

The proposed cloud-edge communication model combines the fast response of edge computing with the storage and computational capabilities of cloud computing. It is suitable for remote patient monitoring, emergency detection and intelligent healthcare systems, particularly when low latency and reliable wireless